HJ50 series

Grip-type single-axis operating handle

Product Features

Heavy-duty industrial electric control handle, specially designed and used for construction machinery industry.

Long-life potentiometer angular displacement detection method.

Both spring return and friction positioning are considered.

A mechanical lock can be carried in the middle position.

A variety of different types of handles are available for selection. At the same time, the number and position of the buttons also have different configurations.

The PWM output directly drives the proportional solenoid valve. And set the starting current, maximum current and PWM frequency according to different proportion solenoid valves.

Optional CAN bus output.

Application range

This series of products are mainly used in cranes, loaders, forklifts, excavators, aerial work vehicles, tractors, harvesters, etc.

Technical Information

Potentiometer type

| Power supply voltage | <36Vdc |
| --- | --- |
| Total resistance value | 200Ω, 1KΩ |
| Electrical angle | ± 18 ° |
| Median voltage | 48% ~ 52% (relative power supply voltage) |
| Median dead zone angle | ± 2.5 ° |
| Potentiometer maximum voltage | 32Vdc |
| Potentiometer allows maximum power consumption | 2W (200Ω), 0.5W (1KΩ) |

Electrical parameters

Direction switch

| Load capacity | 10A @ 30Vdc (resistive load) |
| --- | --- |
| Starting angle | ± 5 ° |
| Contact resistance | <200Ω |

Micro Switch

| Load capacity | 10A @ 30Vdc (resistive load) |
| --- | --- |
| Service life | more than 30 million times (mechanical) |
|  | 200,000 times or more (electrical) |
| Insulation resistance | > 100MΩ(500Vdc insulation resistance meter) |
| Starting angle | ± 3 ° ~ ± 5 ° |

Digital proportional amplifier drive

| Power supply voltage | 9 ~ 36Vdc |
| --- | --- |
| Output drive Two PWM maximum current | 3A |
| PWM frequency | 100HZ ~ 1000HZ |
| Minimum current | 0 ~ 0.4A |
| Maximum current | 0.4 ~ 3A |

Mechanical parameters

| Rocking angle | ± 20 ° |
| --- | --- |
| Mode of operation | automatic spring reset, friction positioning |
| Starting force | 5N |
| Maximum operating force | 11N |
| Limiting force | > 300N |
| Service life | > 2 million times (potentiometer type) |
|  | > 5 million times (Hall type) |
| Weight | 475g (no upper end) |

Environmental parameters

| Working temperature | -30 ℃ ~ + 70 ℃ |
| --- | --- |
| Storage temperature | -40 ℃ ~ + 85 ℃ |
| Protection grade | IP65 (above the installation panel) |

Product selection

series

HJ50

Operation method

Friction positioning

| FCF | median with feel |
| --- | --- |
| FCL | neutral mechanical lock |

Spring return

| SC0 | back to median |
| --- | --- |
| SCL | back to neutral position, neutral mechanical lock |

Note: The mechanical lock is only applicable to the upper end of the DG handle

output signal

| P011 | with 1KQ center tapped potentiometer, 0% ~ 100% Vdc voltage output |
| --- | --- |
| PWM224 | DC24V power supply, output PWM directly drives proportional solenoid valve, maximum output current 3A |

Micro switch (not available when CAN bus is output)

| MS00 | without micro switch |
| --- | --- |
| MS21 | with 10A normally open micro switch |
| MS22 | with 10A front and back position micro switch |

Upper end of handle

| DG | upper end of DG type handle with lifting mechanical lock |
| --- | --- |
| HBR | HBR type handle upper end with rocker on top |
| HDS | HDS type handle upper end, with button on top |
| HDR | top of HDR type handle with rocker on top |
| KGDN | upper end of KG type handle, with safety switch, no rocker button |
| KGDR | upper end of KG type handle, with safety switch, seesaw button |
| SS | For details, please refer to the description of the upper end of the SS handle |

Outgoing way

L direct output (number line)

※ Special selection:

HJ50-20424 and GE-20424 have the same function and are interchangeable;

HJ 50-22BC 1033 and MCH22BC 1033 have the same function and are interchangeable;

HJ 50-12BC 107 and MCH 107 have the same function and are interchangeable;

HJ 50-12BC 1078 and MCH12BC 1078 have the same function and are interchangeable;

Electrical Installations

Line sequence PWM output Potentiometer output

1 9-36V (VCC) front end of potentiometer

2 GND Potentiometer neutral

3 Front direction PWM drive output Potentiometer rear

4 Rear direction PWM drive output Potentiometer sliding end

5 Drive output common terminal Common switch common terminal

6 Switch common terminal Normally open switch

7 Neutral normally open switch Normally open switch in front direction

8 Front direction normally open switch Rear direction normally open switch

9 Rear direction normally open switch